Lewis and Clark National Historical Park

Astoria, Oregon

Septic Tanks and Pitt Toilet Pumping and Maintenace

April 9th, 2023

Prepared By: Kevan Keegan

NATIONAL PARK SERVICE PACIFIC WEST REGION

BACKGROUND

Lewis and Clark National Historical Park, located near Astoria, Oregon, preserves the site of the Lewis and Clark Expedition’s 1805 to 1806 winter encampment at Fort Clatsop and includes several historically and culturally significant locations along the lower Columbia River and Pacific coast. The park encompasses forested landscapes, wetlands, coastal environments, a reconstructed Fort Clatsop, the Fort to Sea Trail, and a number of partner‑managed sites that together interpret the experiences of the Expedition and the Indigenous peoples of the region. Regular and possible emegency septic and vault tank pumping supports ongoing operations at the park.

OBJECTIVE

Vault Toilet Pumping

The park has four vault toilets (CXT brand) each with 1,000 gallon capacity that are pumped at least annually.  Pumping includes removing all solids and liquids from the vault (breaking up solids as necessary) as well as trash that may have been placed in the vault by visitors.  The vaults are located at these locations.

- North Netul:  A four hole pitt toilet. Access by truck with standard hose assemblies possible from pavement.

- South Netul:  A two hole pitt toilet. Access by truck with standard hose assemblies possible from pavement.

- Fort to Sea Parking lot: :  A one hole pitt toilet. Access by truck with standard hose assemblies possible from pavement.

- Skipanon: A one hole pitt toilet. Located approximately a half mile up a narrow gravel road

| Name | Tank Size | Pumping Frequency |
| --- | --- | --- |
| North Netul | 1000 gal | Every two years or as needed |
| South Netul | 1000 gal | Once a Year |
| Fort to Sea | 1000 gal | Twice a Year |
| Skipanon | 1000 gal | Every two years or as needed |

Septic Pumping

The park has 2 individual septic systems with solids tanks ranging in size from 1000 gallons to 10,000 gallons.  In unexpected circumstances (i.e. system failure), an emergency service may be required.

Services Needed

- Pump tanks until empty (break up solids as necessary)

- Clean system filters

- Inspect all pumps

- Inspect, clean and pump, as necessary associated system manhole/junction vaults

- Some of the Septic Sytems have both a solid and small liquid distribution tank for the leach field.

- All tanks are located near enough to pavement that the truck can remain on pavement and the tank pumped with typical length hose assemblies.

Solids Tanks:

- Admin/ Visitor Center 10,000 gallon tank

  - 5,000 gallons pumped annually

  - 10,000 gallons pumped

- Maintenace and Reosuce buildings

  - 1,000 gallon tanks – Pumped As Needed

| Name | Tank Size | Pumping Frequency |
| --- | --- | --- |
| Admin/ Visitor center | 1000 gal | 5k a year 10k every two years |
| Maintenance and resource | 1000 gal | Every 3-5 years |

Emergency call-outs, after acknowledgment of contact, the contractor shall respond within 48 hour notice at a fixed per-gallon rate.